Industrial Monitoring of Humidity Temperature Sensor and Water Level Using Raspberry Pi

:

Component Pin Connections

BME280 (I2C)

VCC	3.3V (Pin 1)

GND	GND (Pin 6)

SDA	GPIO 2 (Pin 3)

SCL	GPIO 3 (Pin 5)

HC-SR04	VCC

5V (Pin 2)

GND	GND (Pin 14)

TRIG	GPIO 23 (Pin 16)

ECHO	GPIO 24 (Pin 18) (with voltage divider)

YF-S201

VCC (Red)	5V (Pin 4)

GND (Black)	GND (Pin 9)

OUT (Yellow)	GPIO 17 (Pin 11)

Relay Module

VCC	5V (Pin 2)

GND	GND (Pin 6)

IN	GPIO 27 (Pin 13)

Buzzer

Signal (+)	GPIO 22 (Pin 15)

GND (-)	GND (Pin 9)

16x2 LCD / OLED (I2C)

VCC	5V (Pin 2)

GND	GND (Pin 6)

SDA	GPIO 2 (Pin 3) (shared)

SCL	GPIO 3 (Pin 5) (shared)

PROGRAMING

import RPi.GPIO as GPIO

import smbus2

import bme280

import time

import csv

import datetime

import pyttsx3

import requests

import sqlite3

from RPLCD.i2c import CharLCD  # Added for LCD

# Initialize text-to-speech engine

tts = pyttsx3.init()

# GPIO pin setup

TRIG = 23

ECHO = 24

FLOW_SENSOR = 17

RELAY = 27

BUZZER = 22

TANK_HEIGHT_CM = 100

I2C_PORT = 1

BME280_ADDRESS = 0x76

# GPIO setup

GPIO.setmode(GPIO.BCM)

GPIO.setup(TRIG, GPIO.OUT)

GPIO.setup(ECHO, GPIO.IN)

GPIO.setup(FLOW_SENSOR, GPIO.IN, pull_up_down=GPIO.PUD_UP)

GPIO.setup(RELAY, GPIO.OUT)

GPIO.setup(BUZZER, GPIO.OUT)

GPIO.output(RELAY, GPIO.HIGH)  # Pump initially off

GPIO.output(BUZZER, GPIO.LOW)

pulse_count = 0

def count_pulse(channel):

global pulse_count

pulse_count += 1

GPIO.add_event_detect(FLOW_SENSOR, GPIO.FALLING, callback=count_pulse)

# BME280 setup

bus = smbus2.SMBus(I2C_PORT)

calibration_params = bme280.load_calibration_params(bus, BME280_ADDRESS)

# LCD setup

lcd = CharLCD('PCF8574', 0x27)  # Adjust I2C address if needed

lcd.clear()

lcd.write_string("Smart Monitor Ready")

time.sleep(2)

def read_distance_cm(timeout=0.03):

GPIO.output(TRIG, False)

time.sleep(0.05)

GPIO.output(TRIG, True)

time.sleep(0.00001)

GPIO.output(TRIG, False)

pulse_start = time.time()

timeout_start = pulse_start

while GPIO.input(ECHO) == 0:

pulse_start = time.time()

if pulse_start - timeout_start > timeout:

return None

pulse_end = time.time()

timeout_start = pulse_end

while GPIO.input(ECHO) == 1:

pulse_end = time.time()

if pulse_end - timeout_start > timeout:

return None

duration = pulse_end - pulse_start

return round(duration * 17150, 2)

def read_bme280():

data = bme280.sample(bus, BME280_ADDRESS, calibration_params)

return data.temperature, data.humidity, data.pressure

def send_telegram_alert(message):

bot_token = 'YOUR_BOT_TOKEN'

chat_id = 'YOUR_CHAT_ID'

url = f'https://api.telegram.org/bot{bot_token}/sendMessage'

payload = {'chat_id': chat_id, 'text': message}

requests.post(url, data=payload)

def send_email_alert(subject, message):

pass

def send_whatsapp_alert(message):

pass

def log_to_db(flow_rate, water_level, temp, humidity, pressure):

conn = sqlite3.connect("sensordata.db")

cursor = conn.cursor()

cursor.execute('''CREATE TABLE IF NOT EXISTS logs (

timestamp DATETIME,

flow_rate REAL,

water_level REAL,

temperature REAL,

humidity REAL,

pressure REAL

)''')

cursor.execute("INSERT INTO logs VALUES (datetime('now'), ?, ?, ?, ?, ?)",

(flow_rate, water_level, temp, humidity, pressure))

conn.commit()

conn.close()

try:

print("Smart Monitoring System Started...")

while True:

pulse_count = 0

time.sleep(1)

flow_rate = pulse_count / 7.5

temp, humidity, pressure = read_bme280()

distance = read_distance_cm()

if distance is None:

print("Ultrasonic timeout")

continue

water_level = max(0, min(100, 100 - (distance / TANK_HEIGHT_CM * 100)))

print(f"Flow Rate: {flow_rate:.2f} L/min")

print(f"Water Level: {water_level:.1f} %")

print(f"Temp: {temp:.2f} °C | Humidity: {humidity:.2f}% | Pressure: {pressure:.2f} hPa")

lcd.clear()

lcd.write_string(f"Temp:{temp:.1f}C\nHumidity:{humidity:.1f}%")

time.sleep(2)

lcd.clear()

lcd.write_string(f"Water:{water_level:.1f}%\nFlow:{flow_rate:.2f}L/m")

if flow_rate > 0 and water_level < 20:

msg = "Leakage Alert: Water is flowing but tank level is low."

print(f"[!] {msg}")

tts.say(msg)

tts.runAndWait()

GPIO.output(BUZZER, GPIO.HIGH)

send_telegram_alert(msg)

send_email_alert("Leak Detected", msg)

send_whatsapp_alert(msg)

elif flow_rate == 0 and water_level < 10:

msg = "Dry Alert: Tank is empty and no water flow."

print(f"[!] {msg}")

tts.say(msg)

tts.runAndWait()

GPIO.output(BUZZER, GPIO.HIGH)

send_telegram_alert(msg)

send_email_alert("Dry Tank Alert", msg)

send_whatsapp_alert(msg)

else:

GPIO.output(BUZZER, GPIO.LOW)

if water_level > 90:

GPIO.output(RELAY, GPIO.HIGH)  # turn OFF pump

elif water_level < 20 and flow_rate > 0:

GPIO.output(RELAY, GPIO.LOW)   # turn ON pump

log_to_db(flow_rate, water_level, temp, humidity, pressure)

except KeyboardInterrupt:

print("Exiting...")

GPIO.cleanup()

SAMPLE OUT[PUT

Smart Monitoring System Started...

Flow Rate: 1.65 L/min

Water Level: 76.8 %

Temp: 28.40 o c I Humidity: 61.80% | Pressure: 1013.05 hPa

LCD:

Temp:28.4C Humidity:61.8% —Y (waits 2 seconds)

Flow:1.65L/m

Flow Rate: 1.80 L/min

Water Level: 18.3 %

Temp: 28.42 o c I Humidity: 62.10% | Pressure: 1013.18 hPa > Leakage Alert: Water is flowing but tank level is low.

> Voice Alert Triggered > Buzzer ON > Telegram Alert Sent > Email Sent (simulated)

> WhatsApp Alert Triggered

LCD:

Temp:28.4C

Humidity:62.1%

Flow:1.80L/m

Flow Rate: 0.00 L/min

Water Level: 9.5 %

Temp: 28.38 o c I Humidity: 61.70% | Pressure: 1013.00 hPa

[!] Dry Alert: Tank is empty and no water flow.

> Voice Alert Triggered > Buzzer ON > Telegram Alert Sent > Email Sent

> WhatsApp Alert Triggered

LCD:

Temp:28.4C

Humidity:61.7%

Flow:O.OOL/m

Flow Rate: 2.05 L/min

Water Level: 92.0 % Pump turned OFF via Relay (GPIO 27)

Flow Rate: 1.35 L/min

Water Level: 14.2 %

Pump turned ON via Relay (GPIO 27)

DATABASE

Timestamp	Flow Rate Water Level Temperature Humidity Pressure

| 2025-06-23 | 1.65 | 76.8 | 28.40 | 61.80 | 1013.05 |
| --- | --- | --- | --- | --- | --- |
| 2025-06-23 | 1.80 | 18.3 | 28.42 | 62.10 | 1013.18 |
| 2025-06 | 11 0.00 | 9.5 | 28.38 | 61.70 | 1013.00 |
| 2025-06-23 | 2.05 | 92.0 | 28.45 | 61.90 | 1013.25 |
| 2025-06-23 | 1.35 | 14.2 | 28.39 | 61.75 | 1013.10 |

Smart Monitoring and Control of Pressure and Power in Toyota Airjet Loom using Arduino

Connection details

Ardiuno UNO Configuration

| Component | Function | Arduino Pin |
| --- | --- | --- |
| MPX5700AP Pressure Sensor | Analog Pressure Input | A0 |
| PZEM-004T | Power Monitoring (Serial UART) | D2 (RX), D3 (TX) |
| 16x2 I2C LCD | Display | A4 (SDA), A5 (SCL) |
| Relay Module | Loom Motor ON/OFF Control | D5 |
| Buzzer | Audio Alert | D6 |
| LED | Visual Alert | D7 |
| Bluetooth Module (HC-05) | Wireless Monitoring | D10 (TX), D11 (RX)* |
| Wi-Fi Module (ESP8266) | Cloud Storage (optional) | TX → D8, RX → D9 (via voltage divider) |
| Power Supply | 12V Adapter or USB | VIN or USB |

MPX5700AP

| Pin on MPX5700AP | Connects to |
| --- | --- |
| VCC | 5V on Arduino |
| GND | GND on Arduino |
| Output (Vo) | A0 on Arduino |

PZEM pin configuratio

| PZEM Pin | Arduino Pin | Notes |
| --- | --- | --- |
| TX | D2 (RX) | Use SoftwareSerial |
| RX | D3 (TX) |  |
| VCC | External 5V | Shared with sensor/buzzer |
| GND | Arduino GND | Common Ground |
| Current Clamp | AC line | Clamp on motor wire |
| Voltage Line | AC Line | Connect Live & Neutral as per manual |

LCD pin configuration

LCD Pin; Arduino Pin. SDA; A4. SCL; A5. VCC; 5V. GND; GND.

Relay pin Configuration

| Relay Pin | Arduino Pin | Description |
| --- | --- | --- |
| IN | D5 | Control signal to toggle relay |
| VCC | 5V | Power for relay |
| GND | GND | Common ground |
| NO/NC/COM | Motor Line | Connect motor AC line through it |

Buzzer Configuration

| Device | Arduino Pin | Notes |
| --- | --- | --- |
| Buzzer | D6 | 5V active buzzer |
| LED | D7 | 220Ω resistor in series |

Bluethooth module

| HC-05 Pin | Arduino Pin | Notes |
| --- | --- | --- |
| VCC | 5V | Power |
| GND | GND | Common Ground |
| TXD | D10 (Arduino RX) | Can go directly |
| RXD | D11 via divider | Use voltage divider (5V → 3.3V) |

HC-05 Configuration

HC-05 Pin; Arduino Pin; Notes. VCC; 5V; Power. GND; GND; Common Ground. TXD; D10 (Arduino RX); Can go directly. RXD; D11 via divider; Use voltage divider (5V → 3.3V).

ESP8266 Configuration

ESP8266 Pin; Arduino Pin; Notes. VCC; 3.3V; Do not connect to 5V!. GND; GND; Common ground. TX; D8 (Arduino RX). RX; D9 (Arduino TX) via divider; 5V → 3.3V via divider. CH_PD/EN; 3.3V; Pull-up resistor may be needed.

Program

// Smart Monitoring and Control of Pressure and Power in Toyota Airjet Loom

// Using Arduino UNO, MPX5700AP, PZEM-004T, LCD, Relay, Buzzer, Bluetooth, and Cloud

#include <LiquidCrystal_I2C.h>

#include <SoftwareSerial.h>

#include <PZEM004Tv30.h>

// LCD setup

LiquidCrystal_I2C lcd(0x27, 16, 2);

// Pressure Sensor (MPX5700AP)

const int pressurePin = A0;

float voltage, pressure_kPa;

float lowerLimit = 400;  // Minimum safe pressure (kPa)

float upperLimit = 600;  // Maximum safe pressure (kPa)

// Actuators

const int relayPin = 5;

const int buzzerPin = 6;

const int ledPin = 7;

// Bluetooth

SoftwareSerial bluetooth(10, 11); // TX, RX

// PZEM Module (Power Monitoring)

SoftwareSerial pzemSerial(2, 3); // RX, TX

PZEM004Tv30 pzem(pzemSerial);

void setup() {

pinMode(relayPin, OUTPUT);

pinMode(buzzerPin, OUTPUT);

pinMode(ledPin, OUTPUT);

lcd.begin();

lcd.backlight();

Serial.begin(9600);

bluetooth.begin(9600);

}

void loop() {

// Pressure calculation

voltage = analogRead(pressurePin) * (5.0 / 1023.0);

pressure_kPa = ((voltage - 0.2) / 4.5) * 700;

// Power parameters

float voltage_p = pzem.voltage();

float current_p = pzem.current();

float power = pzem.power();

float energy = pzem.energy();

float frequency = pzem.frequency();

float pf = pzem.pf();

// Display on LCD

lcd.setCursor(0, 0);

lcd.print("P:"); lcd.print(pressure_kPa); lcd.print("kPa");

lcd.setCursor(0, 1);

lcd.print("V:"); lcd.print(voltage_p); lcd.print(" I:"); lcd.print(current_p);

// Bluetooth Output

bluetooth.print("Pressure: "); bluetooth.print(pressure_kPa); bluetooth.println(" kPa");

bluetooth.print("Voltage: "); bluetooth.println(voltage_p);

bluetooth.print("Current: "); bluetooth.println(current_p);

bluetooth.print("Power: "); bluetooth.println(power);

bluetooth.print("Energy: "); bluetooth.println(energy);

bluetooth.print("Frequency: "); bluetooth.println(frequency);

bluetooth.print("Power Factor: "); bluetooth.println(pf);

// Alert and control logic

if (pressure_kPa < lowerLimit || pressure_kPa > upperLimit || power > 1000) {

digitalWrite(buzzerPin, HIGH);

digitalWrite(ledPin, HIGH);

digitalWrite(relayPin, LOW); // Shut down loom

} else {

digitalWrite(buzzerPin, LOW);

digitalWrite(ledPin, LOW);

digitalWrite(relayPin, HIGH); // Keep loom running

}

delay(2000);

}

Output

1. LCD Display Output (16x2 I2C LCD)

- Normal Condition (Safe Range):

- P:520.3kPa

V:229.4 I:4.2

- Pressure Drop Detected (< 400 kPa):

- P:378.6kPa

⚠ Low Pressure

Overpressure Detected (> 600 kPa):

P:615.7kPa

⚠ High Pressure

2. Bluetooth Serial Output (via HC-05)

- Normal Monitoring Output:

- Pressure: 520.3 kPa

- Voltage: 229.4

- Current: 4.2

- Power: 960.5

- Energy: 3.1

- Frequency: 50.0

Power Factor: 0.95

- On Fault (e.g., low pressure or high power):

- Pressure: 378.6 kPa

- Voltage: 230.1

- Current: 5.2

- Power: 1021.0

Status: FAULT – Loom Shutdown Triggered

3. Buzzer, LED & Relay Status Table

| Condition | Buzzer | LED | Relay (Motor Control) |
| --- | --- | --- | --- |
| Normal Operation | OFF | OFF | ON (Running) |
| Pressure Drop (<400kPa) | ON | ON | OFF (Shutdown) |
| Overpressure (>600kPa) | ON | ON | OFF (Shutdown) |
| Power > 1000W | ON | ON | OFF (Shutdown) |

4. Serial Monitor Output (Arduino IDE)

- Normal State:

- Pressure: 520.3 kPa

- Voltage: 229.4

- Current: 4.2

- Power: 960.5

- Energy: 3.1

- Frequency: 50.0

- Power Factor: 0.95

System Status: NORMAL

- Fault State:

- Pressure: 378.6 kPa

- ⚠ ALERT: Pressure Drop Detected!

System Status: SHUTDOWN