SMART ENERGY METER

CONNECTION:

| ZMPT101B Pin | Connects To (ESP32) | Purpose |
| --- | --- | --- |
| VCC | 5V | Power supply to voltage sensor |
| GND | GND | Common ground |
| OUT | GPIO34 (ADC1_CH6) | AC voltage analog reading |

| ACS712 Pin | Connects To (ESP32) | Purpose |
| --- | --- | --- |
| VCC | 5V | Power supply to ACS712 |
| GND | GND | Common ground |
| OUT | GPIO35 (ADC1_CH7) | AC current analog reading |
| IP+ (Terminal) | LIVE wire input from mains | Current flowing into sensor |
| IP− (Terminal) | LIVE wire output to load | Current flowing out to the load |

| Relay Pin | Connects To (ESP32) | Purpose |
| --- | --- | --- |
| VCC | 5V | Relay power supply |
| GND | GND | Common ground |
| IN | GPIO2 | Control signal from ESP32 |
| COM (Load side) | MAIN Live input | Switching live wire |
| NO/NC | Live output to load | Load control |

| Component | Connects To | Purpose |
| --- | --- | --- |
| ESP32 5V pin | Sensor modules | Power to sensors |
| All GND pins | Common together | Shared reference ground |
| USB cable | ESP32 power | For programming & testing |

| AC Component | Connection | Explanation |
| --- | --- | --- |
| AC Live (Phase) | goes through ACS712 IP+ → IP− | Allows current measurement |
| AC Neutral | goes directly to load | Do not pass neutral through ACS712 |
| AC Live Branch | Connected to ZMPT101B terminal block | Voltage measurement |
| Relay (optional) | Connected in series with Live | For overload cut-off |

ROUGH DIAGRAM:

CODE:

const char* WIFI_SSID = "YOUR_SSID";

const char* WIFI_PASS = "YOUR_PASS";

const char* THINGSPEAK_API_KEY = "YOUR_THINGSPEAK_WRITE_KEY"; // optional, leave "" to disable upload

const bool ENABLE_THINGSPEAK = false; // set true to upload

// Pins

const int V_SENSOR_PIN = 34; // ZMPT101B analog out

const int I_SENSOR_PIN = 35; // ACS712 analog out

const int RELAY_PIN     = 2;  // optional relay

// Sampling and calibration

const unsigned int NUM_SAMPLES = 2000;    // number of ADC samples per measurement window

const unsigned long SAMPLE_INTERVAL_US = 500; // microseconds between samples -> ~2kHz sampling

// ADC reference / conversion (ESP32 ADC characteristics)

const float ADC_REF_VOLTAGE = 3.3;   // approximate

const int ADC_MAX = 4095;           // 12-bit ADC on ESP32 default

// Sensor calibration (set and tune)

float V_CAL = 110.0 / 0.5; // placeholder - we'll calibrate. (example: adjust after initial test)

float I_CAL = 1.0;         // multiply measured current by this factor after calibration

// For ACS712: sensitivity e.g., 185 mV/A for 5A version; if you use ACS712, calibrate in code

// But because ADC scaling & circuit specifics vary, calibrate using known loads.

// Global accumulators

double energy_wh = 0.0; // watt-hours

unsigned long lastEnergyMillis = 0;

// Helper: read analog and convert to voltage

inline float readADCVoltage(int pin) {

int raw = analogRead(pin);

return (raw * ADC_REF_VOLTAGE) / ADC_MAX;

}

void setup() {

Serial.begin(115200);

delay(1000);

pinMode(RELAY_PIN, OUTPUT);

digitalWrite(RELAY_PIN, LOW); // relay OFF by default (change as needed)

// optionally connect wifi if uploading

if (ENABLE_THINGSPEAK && strlen(WIFI_SSID) > 0) {

Serial.print("Connecting to WiFi...");

WiFi.begin(WIFI_SSID, WIFI_PASS);

unsigned long start = millis();

while (WiFi.status() != WL_CONNECTED && millis() - start < 10000) {

delay(200);

Serial.print(".");

}

Serial.println();

if (WiFi.status() == WL_CONNECTED) {

Serial.print("WiFi connected, IP: ");

Serial.println(WiFi.localIP());

} else {

Serial.println("WiFi failed to connect.");

}

}

lastEnergyMillis = millis();

}

void loop() {

// Sample loop

double sumV2 = 0.0;

double sumI2 = 0.0;

double sumP = 0.0;

unsigned int sampleCount = NUM_SAMPLES;

// For offset removal, take midpoints

// For ACS712: midpoint is Vcc/2 (approx). For ZMPT101B module, midpoint may be Vcc/2 too.

float vMid = 0.0;

float iMid = 0.0;

// Quick mid estimate (small sample)

for (int k = 0; k < 50; ++k) {

vMid += readADCVoltage(V_SENSOR_PIN);

iMid += readADCVoltage(I_SENSOR_PIN);

delayMicroseconds(1000);

}

vMid /= 50.0;

iMid /= 50.0;

unsigned long t0 = micros();

for (unsigned int n = 0; n < sampleCount; ++n) {

unsigned long ts = micros();

float vSample = readADCVoltage(V_SENSOR_PIN) - vMid; // centered

float iSample = readADCVoltage(I_SENSOR_PIN) - iMid; // centered

// convert ADC voltage samples to actual volt/current units

// These conversion factors must be calibrated for your sensors and voltage divider/conditioning circuits

// For now we keep in 'voltage units' relative; final calibration uses V_CAL and I_CAL

float vReal = vSample * V_CAL; // V_CAL to be determined experimentally

float iReal = iSample * I_CAL; // I_CAL to be determined experimentally

sumV2 += (double)vReal * (double)vReal;

sumI2 += (double)iReal * (double)iReal;

sumP += (double)vReal * (double)iReal; // instantaneous power (sample-wise averaged)

// wait for next sample

while (micros() - ts < SAMPLE_INTERVAL_US) {

// busy wait

}

}

unsigned long t1 = micros();

double elapsed_s = (double)(t1 - t0) / 1e6;

double Vrms = sqrt(sumV2 / sampleCount);

double Irms = sqrt(sumI2 / sampleCount);

double Pavg = sumP / sampleCount; // average instantaneous power -> real power approx

// Energy accumulation (Wh)

double Wh_increment = (Pavg * (double)elapsed_s) / 3600.0; // watts * sec /3600 = Wh

energy_wh += Wh_increment;

// Print

Serial.println("------------------------------------------------");

Serial.printf("Samples: %u  Time: %.3fs\n", sampleCount, elapsed_s);

Serial.printf("Vrms: %.3f V\n", Vrms);

Serial.printf("Irms: %.3f A\n", Irms);

Serial.printf("Real Power: %.3f W\n", Pavg);

Serial.printf("Energy: %.6f Wh\n", energy_wh);

// Optional: overload cut

double overload_threshold_watts = 2000.0; // set as needed

if (Pavg > overload_threshold_watts) {

Serial.println("**OVERLOAD: turning OFF relay**");

digitalWrite(RELAY_PIN, HIGH); // turn relay ON to cut power if wiring is NO/NC accordingly

} else {

digitalWrite(RELAY_PIN, LOW);

}

// Upload to ThingSpeak every minute (optional)

static unsigned long lastUpload = 0;

if (ENABLE_THINGSPEAK && WiFi.status() == WL_CONNECTED && millis() - lastUpload > 60000) {

lastUpload = millis();

sendToThingSpeak(Vrms, Irms, Pavg, energy_wh);

}

delay(500); // small delay before next measurement

}

void sendToThingSpeak(double Vrms, double Irms, double P, double Wh) {

if (strlen(THINGSPEAK_API_KEY) == 0) return;

if (WiFi.status() != WL_CONNECTED) return;

String url = "http://api.thingspeak.com/update?api_key=" + String(THINGSPEAK_API_KEY);

url += "&field1=" + String(Vrms, 3);

url += "&field2=" + String(Irms, 3);

url += "&field3=" + String(P, 3);

url += "&field4=" + String(Wh, 4);

HTTPClient http;

http.begin(url);

int httpCode = http.GET();

if (httpCode > 0) {

String payload = http.getString();

Serial.printf("ThingSpeak response: %d, %s\n", httpCode, payload.c_str());

} else {

Serial.printf("ThingSpeak failed, error: %d\n", httpCode);

}

http.end();

}

OUTPUT: